Example 14 :  In the given mechanism the crank OA rotates at 200 rpm counter-clockwise. For the given configuration determine.

- Velocity of slider S and angular velocity of the link QB.

- Acceleration of slider S and angular acceleration of the link QB.

Fig. 14.1 Mechanism diagram

| Fig 14.2 velocity diagram | Fig 14.3 acceleration diagram |
| --- | --- |

Diagrams are not to the scale. All dimension in mm.

Steps for velocity diagram :

- vao=2π×200×0.0560=1.047ms

- Scale – 1cm of velocity diagram = 0.1047 m/s

- Mark o,q,guide fix point.

- Take ⊥ to link AO and draw vector from O of 10 cm representing va

- Draw vector vsa ⊥ to link AS from point ‘a’ in velocity diagram, Draw velocity vector for slider wrt guide (horizontal line). at intersection we will get s point.

- Draw vector vbq ⊥ to link QB from fix point and Vbs ⊥ to link BS from point s. at intersection we will get b point.

Velocity of slider = 0.9529 ms       ,     angular velocity of QB = 9.73 rad/s

Steps for acceleration diagram :

- Vao2AO=1.04720.05=21.93 ms2

- Scale – 1 cm of acceleration diagram = 2.193 ms2

- Mark o,q,g fix point.

- Draw 	centrifugal acceleration vector of 10cm ||OA from fix point. Here αoa=0 so tangential acceleration of oa is zero. So mark a1 as a.

- for fasC draw as1 ||AS from a, and from s1 draw ⊥as1 for fast

- draw acceleration vector for slider wrt guide. Intersection of fast and  fsg will give s point in acceleration diagram.

- For fsbC draw sb1 ||SB from s, from b1 draw ⊥sb1 representing fsbt.

- Draw centrifugal acceleration vector qb2 from fix point then draw ⊥

from b2 for tangential component.

- Locate b point at intersection of fsbt and fqbt.

Acceleration of slider S= 17.544 ms2

angular acceleration of QB = 140.352 rad/s^2

| Sr. No. | vector | Magnitude (m/s^2) | direction | sense |
| --- | --- | --- | --- | --- |
| 1. | foaC or oa1 foat or a1a | Vao2AO=1.04720.05=21.93 αoaOA = 0 | ¦¦ OA ⊥ OA | → O At a1 ⊥oa1 |
| 2. | fasC or as1 fast or s1s | Vas2AS=0.72220.2=2.61 αasAS = 15.351 | ¦¦ AS ⊥AS | → A At s1 ⊥as1 |
| 3. | fsbC or sb1 fsbt or b1b | Vsb2SB=0.29320.05=1.719 αsbSB = 4.386 | ¦¦ SB ⊥ SB | → S At b1 ⊥sb1 |
| 4. | fqbC or qb2 fqbt or b2b | Vqb2QB=0.97320.1=9.484 αqbQB = 14.0352 | ¦¦ QB ⊥QB | → Q At b2 ⊥qb2 |
| 5. | fsgC or sg | 8 cm * 2.193 = 17.544 | - | - |

Table 14.1